# APB Verification Architecture in UVM

## APB Overview

The Advanced Peripheral Bus (APB) is part of the AMBA protocol family developed by ARM. APB is optimized for low-power and low-bandwidth peripheral communication. It provides a simple interface with reduced complexity and is mainly used to connect peripherals such as UART, GPIO, Timers, and RTC modules.

The APB protocol operates using a simple state-based transfer mechanism consisting of IDLE, SETUP, and ACCESS phases. It supports read and write transactions with minimal control logic.

# Chapter 1: APB Features

The key features of the APB Protocol are:

- Simple non-pipelined protocol

- Low power peripheral bus

- Separate read and write transfers

- PSTRB based byte-level write masking

- Single master and single/multiple slave support

- Two-phase transfer mechanism

- Address, control, and data phases

- PENABLE based access control

- PREADY based wait-state support

- PSLVERR based error indication

- Synchronous protocol operation

- Easy integration with low-speed peripherals

# Chapter 2: APB Verification Plan

Verification involves studying the APB protocol specification, extracting protocol features to be tested, developing a verification strategy, implementing a verification environment, creating testcases, and achieving functional and code coverage closure.

## 2.1 Feature Extraction

Feature extraction involves identifying protocol features and behaviors that must be verified.

The following APB features are verified:

- Reset behavior

- Idle state verification

- Write transfer verification

- Write byte strobe (PSTRB) verification

- Read transfer verification

- Back-to-back transfers

- Wait-state handling using PREADY

- Error handling using PSLVERR

- Address stability during transfer

- Control signal validation

- PENABLE assertion timing

- PSEL assertion behavior

- Protocol sequencing

- Invalid transaction handling

- Simultaneous read/write prevention

## 2.2 Coverage Plan

Functional coverage is developed to ensure complete verification of APB protocol scenarios.

Coverage includes:

- Read and write operations

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Transfer type coverage

- Wait-state coverage

- Error response coverage

- Back-to-back transaction coverage

- Reset transition coverage

- Cross coverage between operation type and response signals

## 2.3 Checker Plan

The checker plan ensures APB protocol compliance.

Checkers are implemented for:

- Correct APB state transitions

- PENABLE assertion only during ACCESS phase

- PSEL stability during transfer

- Address and control stability during ACCESS phase

- Proper write and read sequencing

- PSTRB valid masking check

- Valid PREADY handling

- PSLVERR response checking

- Read data verification

- Reset signal behavior

## 2.4 Verification Environment Development

The APB verification environment is developed using UVM based layered testbench architecture.

The environment includes:

- Sequence Item (uvm_sequence_item)

- Sequencer and Sequences

- Driver

- Monitor

- UVM Scoreboard Expected Model

- Scoreboard

- Functional Coverage

- UVM Environment (uvm_env)

- UVM Test (uvm_test)

- Interface

- Top module

# Chapter 3: APB Verification Environment Development

## 3.1 APB Verification Architecture

The APB verification environment follows a layered architecture where transactions are generated at high abstraction level and converted into pin-level activity.

Verification flow:

- Sequencer and Sequences creates randomized or directed APB transactions.

- Driver converts transactions into APB bus activity.

- DUT processes APB transactions.

- Monitor samples APB interface signals.

- UVM Scoreboard Expected Model predicts expected outputs.

- Scoreboard compares expected and actual outputs.

- Coverage collector gathers functional coverage.

## 3.2 APB Verification Architecture Diagram

# Chapter 4: APB UVM Verification Components

## 4.1 Sequence Item (uvm_sequence_item)

The Sequence Item (uvm_sequence_item) represents APB transfer information in transaction-level format.

It contains:

- APB address

- Write data

- Read data

- Transfer type

- PWRITE signal

- PSTRB signal

- PSEL signal

- PENABLE signal

- PREADY signal

- PSLVERR signal

### Responsibilities

- Randomization of APB transactions

- Constraint generation

- Protocol field storage

- Communication between components

### Example Fields

class apb_mas_seq_item extends uvm_sequence_item;

rand bit [31:0] paddr;

rand bit [31:0] pwdata;

bit [31:0] prdata;

rand bit [ 3:0] pstrb;

rand bit        pwrite;

bit        psel;

bit        penable;

bit        pready;

bit        pslverr;

endclass

## 4.2 Sequencer and Sequences

The Sequencer and Sequences creates APB stimulus transactions.

It generates:

- Directed transactions

- Random transactions

- Error scenarios

- Wait-state scenarios

- Back-to-back transactions

### Responsibilities

- Random transaction generation

- Constraint control

- Sending sequence items to driver via TLM seq_item_port

- Controlling transaction count

### Sequencer and Sequences Flow

Sequencer and Sequences -> Mailbox -> Driver

## 4.3 Driver

The driver converts transaction-level activity into APB pin-level protocol activity.

The driver receives transactions from the Sequencer and Sequences and drives APB signals through the interface.

### Responsibilities

- Driving APB signals

- Implementing APB SETUP and ACCESS phases

- Applying reset

- Driving read and write operations

- Handling wait states

- Synchronizing with clock

### APB Write Transfer Sequence

- Assert PSEL

- Drive address and write data

- Assert PWRITE

- Assert PENABLE in next cycle

- Wait for PREADY

- Complete transfer

### APB Read Transfer Sequence

- Assert PSEL

- Drive address

- Deassert PWRITE

- Assert PENABLE

- Wait for PREADY

- Sample PRDATA

## 4.4 Monitor

The monitor observes APB bus activity and converts signal-level information into transaction-level data.

The monitor is passive and does not drive any signals.

### Responsibilities

- Sampling APB interface signals

- Collecting read and write transactions

- Detecting protocol activity

- Broadcasting transactions via TLM analysis ports

### Monitor Checks

- PSEL assertion

- PENABLE timing

- Address stability

- Wait-state handling

- Read data capture

## 4.5 UVM Scoreboard Expected Model

The UVM Scoreboard Expected Model predicts expected APB behavior.

It acts as a golden model and maintains expected data for comparison.

### Responsibilities

- Predicting expected read data

- Maintaining memory model

- Handling write updates

- Generating expected transaction results

### UVM Scoreboard Expected Model Operation

- During write transfer:

  - Store data at given address

- During read transfer:

  - Retrieve expected data from stored address

## 4.6 Scoreboard

The scoreboard compares DUT outputs with UVM Scoreboard Expected Model outputs.

It validates protocol correctness and data integrity.

### Responsibilities

- Comparing expected and actual transactions

- Logging PASS/FAIL messages

- Reporting mismatches

- Tracking verification status

### Comparison Checks

- Address match

- Read data match

- Transfer type match

- Error response validation

- Wait-state correctness

### Example Scoreboard Message

WRITE PASS : Address = 0x10 Data = 0x55

READ PASS  : Address = 0x10 Data = 0x55

## 4.7 Functional Coverage

Functional coverage measures verification completeness.

### Coverage Points

- Read operation coverage

- Write operation coverage

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Wait-state coverage

- PSLVERR coverage

- Back-to-back transfer coverage

- Reset coverage

### Cross Coverage

- Read/Write vs Address

- Read/Write vs Wait-state

- Transfer type vs Error response

## 4.8 UVM Environment (uvm_env)

The UVM Environment (uvm_env) instantiates and connects all UVM Verification Components.

### Components instantiated inside environment

- Sequencer and Sequences

- Driver

- Monitor

- UVM Scoreboard Expected Model

- Scoreboard

- Coverage collector

### Responsibilities

- Connecting TLM ports and exports

- Connecting virtual interfaces

- Executing UVM phases (build, connect, run)

- Managing simulation flow

## 4.9 UVM Test (uvm_test)

The UVM Test (uvm_test) controls simulation scenarios.

### Responsibilities

- Creating environment

- Configuring testcases

- Starting random or directed tests

- Applying reset sequence

- Controlling simulation duration

### Example Testcases

- Sanity test

- Write only test

- Read only test

- Random transfer test

- Wait-state test

- Error response test

- Back-to-back transfer test

## 4.10 Interface

The APB interface groups APB signals together and simplifies DUT connectivity.

### APB Interface Signals

interface apb_if(input logic PCLK);

logic PRESETn;

logic PSEL;

logic PENABLE;

logic PWRITE;

logic [31:0] PADDR;

logic [31:0] PWDATA;

logic [31:0] PRDATA;

logic [ 3:0] PSTRB;

logic PREADY;

logic PSLVERR;

endinterface

## 4.11 Why Virtual Interface?

The interface is a static construct, whereas UVM Verification Components such as driver and monitor are dynamic class-based objects.

Since interfaces cannot be directly instantiated inside classes, virtual interfaces are used.

### Definition

A virtual interface is a pointer to an actual interface instance.

virtual apb_if vif;

### Advantages

- Simplifies DUT connection

- Allows class-based access to interface signals

- Improves reusability

- Enables protocol abstraction

# Chapter 5: APB Testcase Flow

## Sanity Test Steps

- Apply reset

- Perform write operation to address 0x10 with data 0x55

- Wait for one clock cycle

- Perform read operation from address 0x10

- Compare read data with expected value

- Report PASS or FAIL

# Chapter 6: Running Simulation

## Compilation

make sanity

make regress

## Simulation

# Chapter 7: Closure Report

Verification closure is achieved after:

- All directed testcases pass

- Random testcases achieve stable results

- Functional coverage reaches target percentage

- Code coverage goals are achieved

- No protocol violations remain

- Scoreboard mismatches are resolved

# References

The APB verification architecture has been developed by referring to the RAM verification architecture document and layered verification methodology structure. fileciteturn0file3